## SIMATS ENGINEERING

### ASSESSMENT TOOL 2

COURSE NAME: COMPUTER NETWORKS COURSE CODE: CSA0713

COURSE OUTCOME COVERED

CO2: Configure IP addressing schemes, subnetting, and basic routing techniques using simulation tool, for efficient network design and topology. (BL3)

Assessment Tool Weightage: 30%

Annexure A

# Scenario-Based

Code of Conduct:

I NS BHAVESH(192565086)(Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

NS BHAVESH(192565086)

Signature of the student:

Annexure A

# Scenario-Based

- A newly established Smart Campus is deploying a secure network infrastructure for its academic and administrative departments. The network administrator has been allocated the IP address block 192.168.100.0/24. Based on the expected number of devices, the following subnet masks have already been assigned:

School of Computing → /25 School of Electronics → /26

School of Mechanical Engineering → /27

As the network engineer, design the subnet allocation by performing the following tasks:

  - Determine	the	network	address	and	broadcast	address	for	each department.

  - Identify the valid host IP address range for each subnet.

  - Calculate the number of usable host addresses available in each subnet.

  - Determine the remaining unused IP address range within the given network block.

- A multinational software company is expanding its headquarters and has been assigned the private IP block 10.10.0.0/16. Different departments require different subnet sizes based on the number of employees. The network administrator has allocated the following subnet masks:

Software Development → /24 Quality Assurance → /25 Human Resources → /26

Executive Management → /27

As the network administrator, perform the following tasks:

  - Determine the network address and broadcast address for each department.

  - Identify the valid range of host IP addresses.

  - Calculate the number of usable hosts in each subnet.

  - Identify the unused IP address range remaining within the allocated IP block.

- A smart hospital is migrating its network infrastructure from IPv4 to IPv6 to support thousands of connected medical devices and IoT sensors. The hospital has been assigned the IPv6 network: 2001:0db8:abcd:0000:0000:0000:0000:0000/64

As the network engineer responsible for the migration, perform the following:

  - Convert the given IPv6 address into its compressed notation.

  - Expand	the	compressed	address	back	into	its	full	hexadecimal representation.

  - Identify the network prefix using the /64 prefix length.

  - Calculate the total number of host addresses supported within the subnet.

  - the total number of possible host addresses available within this network.

- An Internet Service Provider (ISP) connects multiple customer networks through a core router. The router maintains the following routing table entries: 192.168.1.0/24

192.168.2.0/24

192.168.3.0/24

A packet arrives at the router with the destination IP address:

192.168.2.45

As the routing engineer, determine:

  - The destination subnet to which the packet belongs.

### The matching routing table entry.

  - The next-hop network/interface used to forward the packet.

  - The default route that should be used if the destination does not match any routing table entry.

  - A company establishes two office departments connected through a Layer-3 router. The allocated subnet information is as follows:

Administration LAN → 192.168.10.0/26

Finance LAN → 192.168.10.64/26

The router interfaces are configured with:

Administration Gateway → 192.168.10.1

Finance Gateway → 192.168.10.65

As the network administrator, perform the following tasks:

    - Determine the network address and broadcast address for each LAN.

    - Identify the valid host IP address range.

    - Suggest suitable IP addresses for three computers in each LAN.

    - Identify the default gateway that hosts in each LAN should use for communication.

RUBRICS FOR CALCULATION

| Scenario Based Assessment |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Criteria | Weightage (%) | Level 4 (Exemplary) | Level 3 (Proficient) | Level 2 (Developing) | Level 1 (Beginning) |
| Understanding of Scenario | 20% | Demonstrates clear and complete understandin g of the scenario and context | Shows good understandi ng with minor gaps | Partial understandin g; misses key aspects | Misinterpret s or fails to understand the scenario |
| Identification of Key Issues | 20% | Accurately identifies all relevant issues and factors involved | Identifies most key issues with minor omissions | Identifies limited issues; some irrelevant points | Unable to identify key issues |
| Application of Concepts | 25% | Applies appropriate concepts effectively to address the scenario | Applies concepts with minor errors | Limited or partially correct application | Unable to apply relevant concepts |
| Decision Making | 20% | Makes well-reasoned, logical decisions with strong rationale | Makes reasonable decisions with some justification | Makes weak decisions with limited justification | No clear decisions made or rationale provided |
| Clarity of Response | 15% | Response is clear, structured, and logically presented | Generally clear with minor issues | Somewhat unclear or poorly structured | Disorganize d and difficult to understand |

# Question 1 — Smart Campus Subnetting (192.168.100.0/24)

Given: Block 192.168.100.0/24. School of Computing → /25, School of Electronics → /26, School of Mechanical Engineering → /27 (allocated largest-first, VLSM).

## 1 & 2. Network Address, Broadcast Address, and Valid Host Range

| Department | Subnet Mask | Network Address | Broadcast Address | Valid Host Range |
| --- | --- | --- | --- | --- |
| School of Computing | /25 (255.255.255.128) | 192.168.100.0 | 192.168.100.127 | 192.168.100.1 – 192.168.100.126 |
| School of Electronics | /26 (255.255.255.192) | 192.168.100.128 | 192.168.100.191 | 192.168.100.129 – 192.168.100.190 |
| School of Mech. Engg. | /27 (255.255.255.224) | 192.168.100.192 | 192.168.100.223 | 192.168.100.193 – 192.168.100.222 |

## 3. Usable Host Addresses per Subnet

- School of Computing (/25): 2^7 − 2 = 126 usable hosts

- School of Electronics (/26): 2^6 − 2 = 62 usable hosts

- School of Mechanical Engineering (/27): 2^5 − 2 = 30 usable hosts

## 4. Remaining Unused IP Address Range

After allocating the three subnets (192.168.100.0 – 192.168.100.223), the remaining unused block is:

- 192.168.100.224 – 192.168.100.255 (a full /27 block of 32 addresses, reserved for future expansion)

# Question 2 — Multinational Company Subnetting (10.10.0.0/16)

Given: Block 10.10.0.0/16. Software Development → /24, Quality Assurance → /25, Human Resources → /26, Executive Management → /27 (allocated largest-first, VLSM).

## 1 & 2. Network Address, Broadcast Address, and Valid Host Range

| Department | Subnet Mask | Network Address | Broadcast Address | Valid Host Range |
| --- | --- | --- | --- | --- |
| Software Development | /24 (255.255.255.0) | 10.10.0.0 | 10.10.0.255 | 10.10.0.1 – 10.10.0.254 |
| Quality Assurance | /25 (255.255.255.128) | 10.10.1.0 | 10.10.1.127 | 10.10.1.1 – 10.10.1.126 |
| Human Resources | /26 (255.255.255.192) | 10.10.1.128 | 10.10.1.191 | 10.10.1.129 – 10.10.1.190 |
| Executive Management | /27 (255.255.255.224) | 10.10.1.192 | 10.10.1.223 | 10.10.1.193 – 10.10.1.222 |

## 3. Usable Hosts per Subnet

- Software Development (/24): 2^8 − 2 = 254 usable hosts

- Quality Assurance (/25): 2^7 − 2 = 126 usable hosts

- Human Resources (/26): 2^6 − 2 = 62 usable hosts

- Executive Management (/27): 2^5 − 2 = 30 usable hosts

## 4. Unused IP Address Range Remaining

After allocating all four departments (10.10.0.0 – 10.10.1.223 used), the remaining unused address space within the 10.10.0.0/16 block is:

- 10.10.1.224 – 10.10.255.255 (available for future departments/expansion)

# Question 3 — IPv6 Migration for Smart Hospital

Given network: 2001:0db8:abcd:0000:0000:0000:0000:0000 /64

## 1. Compressed Notation

2001:db8:abcd::

(Leading zeros in each group are dropped, and the longest run of consecutive all-zero groups is replaced with '::', used only once in an address.)

## 2. Expanded (Full Hexadecimal) Representation

2001:0db8:abcd:0000:0000:0000:0000:0000

## 3. Network Prefix (/64)

The first 64 bits (first four hextets) identify the network:

Network Prefix = 2001:0db8:abcd:0000::/64

## 4 & 5. Total Number of Host Addresses Supported

A /64 prefix leaves 128 − 64 = 64 bits for the host (Interface ID) portion.

Total possible host addresses = 2^64 = 18,446,744,073,709,551,616 addresses

This enormous address space is more than sufficient to uniquely address every connected medical device and IoT sensor in the hospital, with no need for further subnetting or NAT.

# Question 4 — ISP Core Router Packet Forwarding

Routing table entries: 192.168.1.0/24, 192.168.2.0/24, 192.168.3.0/24

Incoming packet destination IP: 192.168.2.45

## 1. Destination Subnet

192.168.2.45 falls within the range 192.168.2.1 – 192.168.2.254, so it belongs to the 192.168.2.0/24 subnet.

## 2. Matching Routing Table Entry

192.168.2.0/24 — this is the longest/only prefix match for the destination address.

## 3. Next-Hop Network / Interface

The router forwards the packet out of the interface directly connected to the 192.168.2.0/24 network (e.g., the interface configured with an IP such as 192.168.2.1, commonly labeled as the interface serving Customer/Department 2).

## 4. Default Route (No Match Case)

If the destination address did not match any entry in the routing table, the router would forward the packet using the default route 0.0.0.0/0, typically pointing to the ISP's upstream gateway/interface, provided a default route has been configured.

# Question 5 — Layer-3 Router: Administration & Finance LAN

Administration LAN → 192.168.10.0/26, Finance LAN → 192.168.10.64/26

Gateways: Administration → 192.168.10.1, Finance → 192.168.10.65

## 1 & 2. Network Address, Broadcast Address, and Valid Host Range

| LAN | Network Address | Broadcast Address | Valid Host Range |
| --- | --- | --- | --- |
| Administration LAN | 192.168.10.0 | 192.168.10.63 | 192.168.10.1 – 192.168.10.62 |
| Finance LAN | 192.168.10.64 | 192.168.10.127 | 192.168.10.65 – 192.168.10.126 |

## 3. Suggested IP Addresses for Three Computers in Each LAN

Administration LAN (avoiding .1, which is the gateway):

- PC-A1: 192.168.10.2

- PC-A2: 192.168.10.3

- PC-A3: 192.168.10.4

Finance LAN (avoiding .65, which is the gateway):

- PC-F1: 192.168.10.66

- PC-F2: 192.168.10.67

- PC-F3: 192.168.10.68

## 4. Default Gateway for Each LAN

- Administration LAN hosts use default gateway: 192.168.10.1

- Finance LAN hosts use default gateway: 192.168.10.65

Each host must be configured with its respective LAN's gateway address so that traffic destined for the other LAN, or any external network, is forwarded through the Layer-3 router.