SIMATS ENGINEERING

ASSESSMENT TOOL 3

COURSE NAME: COMPUTER NETWORKS

COURSE CODE: CSA0713

COURSE OUTCOME COVERED

CO2: Configure IP addressing schemes, subnetting, and basic routing techniques using simulation tool, for efficient network design and topology. (BL3)

Assessment Tool Weightage: 30%

Dr.V.Parthipan

Annexure A

Debugging & Optimisation Task

Code of Conduct:

I NS BHAVESH(192565086 ) (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

NS BHAVESH(192565086)

Signature of the student:

Annexure A

Debugging & Optimisation Task

- A rapidly growing IT company has shifted its operations to a newly constructed six-storey smart office building. More than 1,200 employees use wireless laptops, smartphones, IP phones, printers, and IoT devices connected through the company's Wireless Local Area Network (WLAN). The office relies heavily on cloud computing, online meetings, video conferencing, and file sharing, making uninterrupted wireless connectivity essential for daily operations. In addition, the company has a team of field engineers and sales executives who travel across different cities and use 4G/5G Wireless Wide Area Network (WWAN) services to access the company's servers, customer databases, and business applications from remote locations.

During the last few weeks, employees have reported several connectivity issues. Users experience slow internet speeds during peak working hours, frequent Wi-Fi disconnections while moving between floors, and weak wireless signals in conference rooms. Video conferencing applications often freeze or disconnect, affecting communication with international clients. Network administrators have also observed that some wireless access points are overloaded while others remain underutilized, leading to uneven traffic distribution. Furthermore, field engineers working in rural and highway areas frequently lose WWAN connectivity, delaying the upload of maintenance reports and customer information. Security audits reveal that a few wireless devices are still connected using outdated security protocols, exposing sensitive company information to potential cyber threats. The management has requested the network engineering team to investigate these issues, identify the root causes, and optimize both the WLAN and WWAN infrastructure to ensure high performance, secure communication, and seamless connectivity for all users.

- A multinational financial organization operates its headquarters, regional offices, and multiple data centers using an Asynchronous Transfer Mode (ATM) network to support mission-critical services such as online banking, video conferencing, Voice over IP (VoIP), and real-time financial transactions. The ATM network uses fixed-size 53-byte cells and establishes Virtual Circuits (VCs) to provide reliable and predictable Quality of Service (QoS). Recently, the organization has experienced several performance issues during peak business hours. Customers report delays in online transactions, voice calls suffer from jitter and occasional packet loss, and video conferencing sessions experience reduced quality. Network monitoring indicates that several ATM switches are operating at high utilization, some Virtual Circuits are congested, and available bandwidth is not being efficiently allocated among different traffic classes. Engineers also discover that certain applications are using inappropriate ATM service categories, leading to increased delay and inefficient resource utilization. The IT department has assigned you as the network engineer to investigate the ATM network, identify the faults, debug the performance issues, and recommend optimization strategies to restore efficient, reliable, and high-quality communication across the enterprise.

- A multinational e-commerce company has interconnected its headquarters, regional offices, warehouses, and cloud data centers using a Multiprotocol Label Switching (MPLS) network to ensure fast, secure, and reliable communication. The MPLS infrastructure supports critical business applications, including online order processing, enterprise resource planning (ERP), video conferencing, Voice over IP (VoIP), cloud-based inventory management, and real-time customer support. Recently, the organization has experienced frequent performance issues during peak business hours. Employees report slow access to cloud applications, delayed order processing, poor-quality voice calls, and interruptions during video conferences. Network monitoring reveals that some Label Switched Paths (LSPs) are heavily congested, while other paths remain underutilized. The IT team also discovers that several routers have incorrect label mappings, causing inefficient routing and increased latency. In addition, traffic engineering has not been properly configured, resulting in uneven bandwidth utilization across the MPLS backbone. Since these issues are affecting customer satisfaction and business operations, the network administrator has asked you to investigate the MPLS network, identify the faults, debug the performance problems, and recommend suitable optimization techniques to improve the overall efficiency and reliability of the network.

- A multinational logistics company operates an automated warehouse where barcode scanners, RFID readers, autonomous robots, and inventory management systems continuously exchange data over a computer network. Every day, thousands of shipment records, customer details, and inventory updates are transmitted between warehouse terminals and the central server. Recently, the company has observed that some shipment records are arriving with incorrect information, barcode scans occasionally fail, and inventory quantities in the database do not always match the physical stock. During peak hours, network monitoring reports an increase in transmission errors caused by electrical interference, signal attenuation, and network congestion. Although some corrupted frames are detected and retransmitted, others remain undetected, resulting in incorrect database entries and delayed order processing. The IT department suspects that the current error detection and correction mechanisms are not configured efficiently. As the network engineer, you are assigned to investigate the transmission errors, identify the weaknesses in the existing error control techniques, and recommend suitable debugging and optimization strategies to improve the reliability and accuracy of data communication.

- A leading online education platform conducts live classes and online examinations for more than 15,000 students across different locations. During examinations, the central server simultaneously sends question papers, images, videos, and answer submission acknowledgements to all students over the network. The server is equipped with a high-speed 10 Gbps connection, whereas many students access the system using slower broadband or mobile internet connections. During peak hours, students experience incomplete downloads, delayed page loading, duplicate packets, and repeated retransmissions. Some students receive the next set of data before their devices finish processing the previous one, causing receiver buffer overflow and application crashes. Network administrators also observe an excessive number of retransmissions, increased latency, reduced throughput, and poor bandwidth utilization. The current flow control mechanism is not effectively regulating the transmission rate between the sender and receivers. As the network engineer, you have been assigned to investigate the communication problems, identify the causes of inefficient flow control, debug the existing system, and recommend suitable optimization techniques to ensure reliable and efficient data transmission during high-traffic periods.

RUBRICS FOR CALCULATION

| Criteria | Weightage | Level 4 (Exemplary) | Level 3 (Proficient) | Level 2 (Developing) | Level 1 (Beginning) |
| --- | --- | --- | --- | --- | --- |
| Problem Identification | 20% | Accurately identifies all errors and root causes with clear understanding | Identifies most errors with minor gaps | Identifies some errors but misses key issues | Unable to identify errors correctly |
| Debugging | 25% | Applies systematic debugging techniques effectively to resolve issues | Uses appropriate debugging methods with minor errors | Limited debugging approach; requires guidance | Unable to debug or resolve issues |
| Optimization | 20% | Optimizes code by significantly improving time complexity using efficient algorithms | Improves time complexity with minor gaps in efficiency | Limited improvement in time complexity | No improvement; inefficient approach retained |
| Analysis | 20% | Demonstrates strong logical reasoning and clear justification of solutions | Shows reasonable analysis with some justification | Limited analysis; weak reasoning | No analytical reasoning demonstrated |
| Code Quality | 15% | Produces clean, well-structured code with proper comments and documentation | Code is mostly clear with minor documentation gaps | Code lacks structure and proper documentation | Poorly written code with no documentation |

# Question 1 — Smart Office WLAN & WWAN Optimization

Scenario: A six-storey smart office with 1,200+ employees on WLAN, plus field staff on 4G/5G WWAN, is facing slow speeds, roaming disconnects, weak conference-room signal, video-call freezes, uneven AP load, rural WWAN drop-outs, and outdated wireless security.

## 1. Problem Identification (Root Causes)

- Insufficient AP density and poor AP placement, leaving coverage gaps between floors and in conference rooms (signal attenuation through concrete/steel).

- No structured channel plan — overlapping/co-channel interference between neighbouring APs on the same 2.4/5 GHz channels.

- High client density per AP during peak hours causing airtime contention and throughput drop.

- No load balancing or band steering, so some APs are overloaded while nearby APs sit idle.

- Lack of fast/seamless roaming support, causing drops as users move between floors.

- WWAN signal loss in rural/highway zones is a carrier coverage limitation, worsened by using a single carrier with no failover.

- Some endpoints still use outdated WEP/WPA-Personal instead of WPA2/WPA3-Enterprise, exposing traffic to interception.

## 2. Debugging Approach

- Conduct a full wireless site survey (predictive + on-site heat map) per floor to locate dead zones and interference.

- Pull AP-level analytics from the WLAN controller: client count, channel utilization, retry/roam-failure rates.

- Audit channel and transmit-power settings on each AP for overlap/co-channel conflicts.

- Run a NAC/security audit to list devices still authenticating with legacy protocols.

- Collect WWAN signal-strength (RSRP/RSRQ) logs from field devices at the reported low-coverage locations.

## 3. Optimization Recommendations

- Add APs (or reposition existing ones) per the survey heat map, with dedicated coverage for conference rooms and floor-edge zones.

- Design a non-overlapping channel plan (e.g., 20 MHz channels on 5 GHz with proper reuse pattern) and enable auto-RF/power control.

- Enable band steering and client/AP load balancing so clients are steered to the least-congested AP/band.

- Enable 802.11r/k/v (fast BSS transition, neighbor reports, roam triggers) for seamless inter-floor roaming.

- Migrate all devices to WPA3-Enterprise (802.1X) and retire legacy security protocols via NAC enforcement.

- Apply WMM/DSCP-based QoS to prioritize video-conferencing traffic over bulk downloads.

- For WWAN: deploy dual-SIM/multi-carrier routers with automatic failover, and consider SD-WAN to aggregate LTE/5G links for field engineers in weak-coverage areas.

## 4. Analysis / Justification

- Fixing coverage and channel planning first addresses the root physical-layer cause of drops and weak signal, rather than just adding bandwidth.

- Load balancing and fast roaming target the uneven-AP-utilization and hand-off-freeze symptoms directly.

- Security upgrade (WPA3-Enterprise) is prioritized because it closes the identified data-exposure risk without affecting performance.

- Multi-carrier WWAN failover is the most reliable fix for geographically-caused signal loss, since no single carrier covers all rural/highway routes.

# Question 2 — ATM Network Debugging & QoS Optimization

Scenario: A financial organization's ATM (53-byte cell, Virtual Circuit) network is showing transaction delays, VoIP jitter/loss, poor video quality, high switch utilization, VC congestion, and misused ATM service categories during peak hours.

## 1. Problem Identification (Root Causes)

- Applications are mapped to the wrong ATM service category — e.g., voice/video likely running over UBR/ABR (best-effort) instead of CBR/rt-VBR, so they get no delay/jitter guarantee.

- No (or weak) Connection Admission Control (CAC), allowing switches to accept more VCs than available bandwidth supports.

- Congested Virtual Circuits during peak hours indicate inadequate traffic shaping/policing at the network edge.

- Bandwidth is statically allocated rather than dynamically managed across traffic classes, so utilization is uneven.

## 2. Debugging Approach

- Monitor Cell Loss Ratio (CLR), Cell Delay Variation (CDV), and Cell Transfer Delay per VC to pinpoint congested circuits.

- Check the ATM service category (CBR, rt-VBR, nrt-VBR, ABR, UBR) assigned to each application/VC.

- Review switch CPU, buffer occupancy, and port utilization during peak-hour windows.

- Verify traffic contract parameters — Peak Cell Rate (PCR), Sustainable Cell Rate (SCR), Cell Delay Variation Tolerance (CDVT) — against actual traffic profiles.

## 3. Optimization Recommendations

- Reassign service categories correctly: CBR for VoIP (fixed bandwidth, minimal delay/jitter), rt-VBR for interactive video, nrt-VBR/ABR for transactional banking data, UBR only for pure best-effort traffic.

- Implement/strengthen CAC so new VCs are only admitted if bandwidth can be guaranteed, preventing oversubscription.

- Apply traffic shaping at ingress switches to smooth bursty traffic before it enters the core.

- Increase PCR/SCR allocation on chronically congested VCs, or add parallel VCs/paths for load distribution.

- Upgrade or add redundant ATM switches on high-utilization links, and re-balance VC routing across the backbone.

## 4. Analysis / Justification

- Correct service-category mapping is the highest-leverage fix — it directly targets why voice/video suffer while data does not, without needing new hardware.

- CAC and traffic shaping prevent recurrence of congestion rather than just relieving it once, giving a sustainable QoS guarantee for real-time financial transactions.

# Question 3 — MPLS Network Fault Debugging & Traffic Engineering

Scenario: An e-commerce company's MPLS backbone connects HQ, regional offices, warehouses, and cloud data centers. Some Label Switched Paths (LSPs) are congested while others are underused, some routers have incorrect label mappings, and traffic engineering is not configured — causing latency and poor voice/video/ERP performance.

## 1. Problem Identification (Root Causes)

- LSPs are likely established using default IGP shortest-path routing without MPLS Traffic Engineering (RSVP-TE), so multiple flows pile onto the same 'shortest' path while alternate paths sit idle.

- Incorrect label mappings on some routers point to misconfigured or stale LDP bindings, causing packets to take inefficient or looping paths.

- No bandwidth-aware path selection (no CSPF/traffic-engineering database in use), so link utilization is not considered when placing LSPs.

- No traffic prioritization between latency-sensitive (VoIP, video) and best-effort (file transfer) classes on the shared LSPs.

## 2. Debugging Approach

- Use MPLS LSP ping/traceroute to verify the actual path and label switching of each LSP end-to-end.

- Inspect the Label Forwarding Information Base (LFIB) on each router to confirm label bindings match the intended path.

- Check LDP/RSVP-TE session status and neighbor adjacencies for stale or broken sessions causing wrong bindings.

- Monitor per-LSP bandwidth utilization and compare against physical link capacity to confirm which LSPs are congested vs. underused.

## 3. Optimization Recommendations

- Enable MPLS Traffic Engineering (RSVP-TE) with CSPF so LSPs are placed based on real-time link utilization, not just IGP metric.

- Correct/refresh label bindings by resetting affected LDP sessions and validating LFIB entries after reconvergence.

- Configure auto-bandwidth on TE tunnels so LSP reservations adjust automatically to actual traffic demand.

- Use ECMP or multiple parallel LSPs to spread load across underutilized links.

- Deploy MPLS Fast Reroute (FRR) for sub-50ms failover on critical paths.

- Apply DiffServ-aware Traffic Engineering (DS-TE) to give VoIP/video LSPs strict priority over ERP/bulk-data LSPs.

## 4. Analysis / Justification

- Moving from plain IGP routing to RSVP-TE with CSPF is the core fix — it resolves the uneven-utilization symptom because path placement finally considers available bandwidth.

- Correcting label mappings is treated as a distinct, more immediate fix since it is an active misconfiguration causing latency right now, separate from the longer-term TE rollout.

- DS-TE ensures the optimization does not just balance links but also protects latency-sensitive traffic quality, matching the business-critical nature of VoIP and video.

# Question 4 — Warehouse Network Error Control Debugging

Scenario: An automated warehouse's barcode scanners, RFID readers, robots, and inventory systems are producing corrupted shipment records, failed scans, and stock mismatches, especially at peak hours, due to electrical interference, attenuation, and congestion — with some corrupted frames going undetected.

## 1. Problem Identification (Root Causes)

- Weak error-detection coverage — likely a simple parity check or basic checksum is in use, which cannot reliably detect burst/multi-bit errors caused by electrical interference.

- Signal attenuation and RF/electrical noise (motors, robots, dense metal shelving) are corrupting frames at the physical layer.

- Network congestion during peak hours increases collision/queueing-related frame loss and retransmission delay.

- Some corrupted frames pass undetected because the current error-detection scheme has poor coverage for the error patterns actually occurring (e.g., burst errors), letting bad data reach the database.

## 2. Debugging Approach

- Measure frame/bit error rate at the data link layer during peak vs. off-peak hours to correlate errors with congestion and interference sources.

- Review the current checksum/parity/CRC implementation and its Hamming distance/error-coverage capability.

- Physically inspect cabling, shielding, and wireless channel assignment near robots/motors for RF/electrical noise sources.

- Audit the ARQ (retransmission) protocol's timers and retry logic to see why some errors are not triggering retransmission.

## 3. Optimization Recommendations

- Upgrade error detection from parity/simple checksum to CRC-32, which has much stronger burst-error detection capability suited to noisy industrial environments.

- Add Forward Error Correction (e.g., Hamming code) on critical real-time links so common single/double-bit errors are corrected without needing a retransmission round-trip.

- Use shielded/twisted-pair cabling or move wireless scanners to a less congested, interference-resistant channel/band away from motor-generated noise.

- Implement Selective-Repeat ARQ instead of a coarser Go-Back-N or undetected-loss scheme, so only the specific corrupted frame is retransmitted, improving efficiency during peak congestion.

- Add an application-layer checksum/hash validation on the final database write as a last line of defence for end-to-end data integrity.

## 4. Analysis / Justification

- Switching to CRC-32 directly targets the 'undetected corrupted frames' symptom, since CRC has far higher error-detection probability than parity/simple checksums.

- Adding FEC reduces reliance on retransmission during congestion, improving both accuracy and peak-hour throughput simultaneously.

- Selective-Repeat ARQ is chosen over Go-Back-N because it minimizes redundant retransmission — important when bandwidth is already constrained at peak hours.

# Question 5 — Online Exam Platform Flow-Control Debugging

Scenario: An education platform serves 15,000+ students from a 10 Gbps server, while students use much slower broadband/mobile links. During exams this causes incomplete downloads, delayed loading, duplicate packets, repeated retransmissions, receiver buffer overflow, and app crashes — pointing to ineffective flow control between sender and receivers.

## 1. Problem Identification (Root Causes)

- Large sender/receiver bandwidth mismatch (10 Gbps server vs. slow client links) is not being compensated for by adaptive flow control.

- The sender is likely using a fixed/oversized transmission window that ignores the receiver's actual advertised window (rwnd), causing buffer overflow on slower clients.

- Lack of proper congestion control means the server keeps sending at a rate the network/receiver path cannot sustain, producing duplicate packets and repeated retransmissions.

- No pacing/rate-shaping at the server to match each client's real-time capacity, so faster and slower clients are treated identically.

## 2. Debugging Approach

- Capture and analyze TCP traffic to check whether the receiver's advertised window (rwnd) is actually being respected by the sender.

- Look for duplicate ACKs, retransmission timeouts (RTO), and out-of-order segments in packet captures during peak exam load.

- Compare server burst/chunk size against typical client buffer sizes to quantify overflow risk.

- Identify which congestion-control algorithm is active on the server and whether it adapts to slow/variable-bandwidth clients.

## 3. Optimization Recommendations

- Enforce proper sliding-window flow control so the server strictly honours each receiver's advertised window size, preventing buffer overflow.

- Adopt a congestion-control algorithm suited to variable/mixed bandwidth clients (e.g., BBR or CUBIC) instead of a fixed-rate send.

- Apply server-side rate limiting/traffic shaping per client session so transmission speed is paced to each client's measured capacity.

- Use chunked/paced content delivery (and adaptive bitrate for video/media content) instead of bulk transmission, so slower devices are not overwhelmed.

- Deploy a CDN or regional caching/load-distribution for exam content to reduce the direct sender-to-receiver bandwidth gap.

## 4. Analysis / Justification

- Strict rwnd enforcement is the direct fix for the buffer-overflow and app-crash symptom, since it stops the sender from exceeding what the client can absorb.

- Switching congestion-control algorithms and adding pacing addresses the duplicate-packet and excessive-retransmission symptoms by matching send rate to real network conditions rather than reacting after loss occurs.

- CDN/regional distribution is recommended as a scalability-level optimization, since 15,000 simultaneous students concentrated on one 10 Gbps origin server will remain a bottleneck even after flow-control tuning.